FB21
Firefighting Thermal Camera

The HIKMICRO FB21 is a compact, reliable, firefighting thermal camera designed for firefighters to observe in total darkness and smoke, helping to save lives, protect property, and keep firefighters safe.

Features like dual cameras, LED flashlights, easy-to-navigate buttons, and a temperature range of up to 550°C (1022°F) help you work effortlessly in harsh environments. 8 color palettes for you to choose from, which satisfy most application scenarios.

Key Feature

| - Thermal resolution: 256 x 192 (49 152 pixels) |  |
| --- | --- |
| - NETD : < 40 mK (@ 25 °C, F#=1.0) |  |
| - Temperature measurement range : -20°C to 550°C (-4°F to 1022°F） |  |
| - Up to 10 minutes for ambient temperature -10°C to 90°C (14°F to 194°F) ; Up to 2 minutes for ambient temperature -10°C to 115°C (14°F to 239°F) |  |
| - Measurement presets: Center spot, Hot spot, Cold spot, User Preset Point |  |
| - 25 Hz image frequency |  |
| - 3.2'' LCD screen display |  |
| - Up to 6 hours continuous running - Support object details enhancement |  |

| - Specification |
| --- |
| Infrared Image |  |
| IR Resolution | 256 × 192 (49,152 pixels) |
| NETD | < 40 mK (@ 25 °C, F#=1.0) |
| Image Frequency | 25 Hz |
| Detector Pitch | 12 μm |
| Spectral Range | 7.5 to 14 μm |
| Focal Length | 3.6 mm |
| F-number | F1.0 |
| Field of View (FOV) | 37.2° × 50.0° |
| Spatial Resolution (IFOV) | 3.3 mrad |
| Min. Focus Distance | 0.3 m (0.98 ft) |
| Focus Mode | Focus Free |
| Image Display |  |
| Visual Camera | 1600 × 1200 (2 MP) |
| Display | 240 × 320 Resolution, 3.2”LCD Screen |
| Color Alarm | Above alarm |
| Color Palettes | Basic, Black Hot, White Hot, Fire Detection, Rescue, Hot Area, Cold Area, Building |
| Image Modes | Thermal/Visual/Fusion/PIP |
| Measurement and Analysis |  |
| Object Temperature Range | -20°C to 550°C (-4°F to 1022°F) |
| Accuracy | Max. (± 2°C/3.6°F, ± 2%), for ambient temperature 15°C to 35°C (59°F to 95°F) and object temperature above 0°C (32°F) |
| Measurement Tools | Center Spot, Hot Spot, Cold Spot User-definable: 3 spots |
| Data Storage and Communication |  |
| Storage Media | Built-in 16 GB flash memory |
| Image Storage Capacity | Approx. 90,000 Images |
| General |  |
| USB Interface | USB Type-C |
| LED Light | Yes |
| Battery Type | Rechargeable Li-ion battery |
| Battery Charging Time | Approx. 3 hours fully charged |
| Battery Operating Time | Approx. 6 hours |
| Protection Level | IP65 |
| Drop Test Height | 2 m (6.56 ft) |
| Working Temperature Range | Up to 10 minutes for ambient temperature -10°C to 90°C (14°F to 194°F); up to 2 minutes for ambient temperature -10°C to 115°C (14°F to 239°F) |
| Storage Temperature Range | -20°C to 60°C (-4°F to 140°F) |
| Relative Humidity | < 95% non-condensing |
| Weight | Approx. 380 g (0.84 lb) |
| Dimension | 221.7 × 73.5 × 123.8 mm (8.73 × 2.89 × 4.87 in) |
| Tripod Mounting | UNC ¼”-20 |

- Available Model FB21

- Dimension

Unit: mm [inch]

COMPLIANCE NOTICE: The thermal series products might be subject to export controls in various countries or regions, including without limitation, the United States, European Union, United Kingdom and/or other member countries of the Wassenaar Arrangement. Please consult your professional legal or compliance expert or local government authorities for any necessary export license requirements if you intend to transfer, export, re-export the thermal series products between different countries.