HM-TP42-3AQF/W-Pocket2

Pocket Thermography Camera

HIKMICRO Pocket Series is equipped with a high-resolution HIKMICRO VOx detector, an 8 MP optical camera, and a 3.5” LCD touch screen. It helps you quickly and efficiently locate the hidden problems and accurately measure abnormal temperature. The wide temperature range of -20°C to 400°C is ideal for building inspection, HVAC, electrical, and mechanical equipment maintenance. The 25 Hz frame rate keeps up with fast inspections, and the Wi-Fi function allows the Pocket to share images in real time. The Pocket Series is a powerful thermography tool in your pocket, so it is always there whenever you need it.

- Thermal resolution: 256 × 192 (49,152 pixels), NETD: < 40 mK (@ 25°C, F# = 1.0)

- Accurately measure temperature from -20°C to 400°C (-4°F to 752°F)

- Accuracy: Max. (± 2°C/3.6°F, ± 2%)

- 25 Hz image frequency

- Thermal, fusion, PIP, and optical modes, 8 MP Optical camera

- Manual, automatic, and 1-Tap level and span

- High temperature alarm

- Built-in 16 GB memory

- LED work light

- 1.0x to 4.0x continuous digital zoom

- Up to 4 hours continuous running

| Thermal Module |  |
| --- | --- |
| IR Resolution | 256 × 192 (49, 152 pixels) |
| Spectral Range | 8 to 14 μm |
| NETD | < 0.04°C (40 mK) at 25°C (77°F) ambient temperature, F# = 1.0 |
| Focal Length | 3.5 mm |
| Focus | Fixed |
| FOV (Field of View) | 50° H × 37.2° V |
| IFOV (Spatial Resolution) | 3.43 mrad |
| Image Frequency | 25 Hz |
| Min. Focusing Distance | 0.3 m (0.98 ft) |
| Detector Type | Vanadium Oxide Uncooled Focal Plane Array |
| Detector Pitch | 12 μm |
| F-number | F1.1 |
| optical Module |  |
| FOV (Field of View) | 64.6° H × 51.5° V |
| Picture Resolution | Configurable: 2 MP, 5 MP, 8 MP |
| Video Resolution | 640 × 480 |
| Focus | Fixed |
| Built-in Visual Camera | 3264 × 2448 (8 MP) |
| image display |  |
| Level/ Span | Auto/Manual/1-Tap Touch-screen |
| Display | 640 × 480 Resolution, 3.5” LCD Touch Screen |
| Image Modes | Thermal/Optical/Fusion/PIP |
| Color Palettes | 7: White Hot, Black Hot, Rainbow, Ironbow, Red Hot, Fusion, Rain |
| Fusion | Thermal image with visual camera details |
| Digital Zoom | 1.0x to 4.0x continuous |
| PIP | Resizable and movable infrared area on the visual image |
| Screen Brightness | Manual adjustment |
| Minimum Temp Span (Manual) | 2°C (3.6°F) |
| measurement and analysis |  |
| Object Temperature Range | -20°C to 400°C ( -4°F to 752°F ) |
| Accuracy | Max. (± 2°C/3.6°F, ± 2%), for ambient temperature from 15°C to 35°C (59°F to 95°F) and object temperature above 0°C (32°F) |
| Measurement Presets | Center Spot, Hot Spot, Cold Spot, Off |
| High Temperature Alarm | Audible/visual alarm of above temperature threshold, user-defined temperature threshold |
| On-Camera Emissivity Correction | Variable from 0.01 to 1.0 or selected from the materials list |
| On-Camera Reflected Temperature Compensation | Automatic, based on the input of the reflected temperature |
| On-Camera Transmission Correction | Automatic, based on the inputs for distance and humidity |
| storage |  |
| Storage Media | Built-in EMMC (16 GB) |
| Image Storage Capacity | Approx. 60,000 images |
| Image File Format | Radiometric.JPEG with measurement data included Thermal.THM.JPEG without measurement data included |
| Video Storage Capacity | Approx. 10 h |
| Video File Format | Standard MP4 non-radiometric video |
| Text Note | Max. 200 Characters |
| power system |  |
| Power Supply | 5 VDC/2 A (Charging via USB), fast charging |
| Power Consumption | 1.92 W |
| Battery Type | Rechargeable Li-ion battery |
| Battery Operating Time | 4 hours continuous running per battery (actual life varies depending on settings and usage) |
| Battery Charging Time | 1.5 h to 94%, 2.5h to full charge |
| Battery Charging System | In-imager charging, fast charging |
| AC operation | AC operation with included power supply (100 VAC to 240 VAC, 50/60 Hz) |
| General |  |
| Wi-Fi | 802.11 b/g/n (2.4 GHz, 5 GHz) |
| Bluetooth | Bluetooth 4.2 |
| USB Interface | USB 2.0, USB Type-C |
| LED light (torch) | Yes |
| Microphone | Yes |
| Buzzer | Yes, audible alarm of above temperature threshold |
| Carrying pouch | Yes |
| Warranty | 3 years for whole product, 10 years for detector, and 2 years for battery |
| Recommended calibration cycle | 1 year (Assumes normal operation and normal aging) |
| Safety | IEC 62368-1: Overvoltage category II, Pollution Degree 2 |
| Language | 21 Languages English, German, French, Spanish (Spain), Portuguese (Portugal), Italian, Czech, Slovak, Polish, Hungarian, Romanian, Dutch, Danish, Norwegian, Finnish, Swedish, Russian, Turkish, Japanese, Korean, Chinese (Traditional) |
| Working Temperature Range | -10°C to 50°C (14°F to 122°F) |
| Protection level | IP54 (protected against dust, limited ingress; protection against water spray from all directions) |
| Drop Test Height | 2 m (6.56 ft) |
| Weight | Approx. 218 g (0.481 lb) |
| Dimension | 138.5 mm × 85.2 mm × 23.6 mm (5.45” × 3.35” × 0.93”) |
| Storage Temperature Range | -20°C to 60°C (-4°F to 140°F) |
| Tripod Mounting | UNC ¼”-20 |
| Relative Humidity | < 95% non-condensing |
| Electromagnetic compatibility | EN55032: 2015+A11: 2020 EN50130-4: 2011+A1: 2014 EN61000-3-3: 2013 |
| Australian RCM | EN 61010-1: 2010 EN55032: 2015+A11: 2020 EN50130-4: 2011+A1: 2014 EN61000-3-3: 2013 |
| US FCC | CFR 47, Part 15 Subpart B |
| RoHS compliant | Yes |
| Vibration | 0.03 g2/Hz (3.8 g), 2.5 g IEC 60068-2-6 |
| Shock | 25 g, IEC 68-2-29 |
| software |  |
| PC Software | HIKMICRO Analyzer |
| APP | HIKMICRO Viewer App |
| Cast Screen | UVC Alarm Client |

HM-TP42-3AQF/W-Pocket2

| Camera (× 1) | Quick Start Guide (× 1) | Power Supply (× 1) |  | USB Cable (× 1) |
| --- | --- | --- | --- | --- |
| Carrying Pouch (× 1) | Wrist Strap (× 1) |  |  |  |

COMPLIANCE NOTICE: The thermal series products might be subject to export controls in various countries or regions, including without limitation, the United States, European Union, United Kingdom and/or other member countries of the Wassenaar Arrangement. Please consult your professional legal or compliance expert or local government authorities for any necessary export license requirements if you intend to transfer, export, re-export the thermal series products between different countries.